aHow 2 fit an Omega Wiper Switch

The Omega wiper switch can be fitted to the Cavalier & Calibra.

http://www.akirwan.freeserve.co.uk/akhow2/omegaw.jpg

This provides 2 benfits.

- Variable time Intermittant wiper control

- Fuel computer buttons on the end

For a cavalier saloon you will require an Omega Saloon Wiper Switch.

For a hatch you will require an Omega Estate Wiper Switch.

An Astra GSi or sport has fuel computer buttons on the end but without the variable wiper intermittant control.

You will also require an Omega wiper relay for the variable intermittant.

This is a 6 pin relay located in the fusebox part number 90 494 187.

It has 1 small spade terminal and 5 normal spade terminals not to be confused with the other 6 pin relay in there which has 2 large spade terminals, 3 normal spades and 1 small as this is the heated rear window relay.

The  wiper switch has an additional connector. You can either cut this off or obtain the other half from a scrap yard.

In the passenger footwell of a Cav 3 are some coloured 9 pin connectors. Cut out the black one and this will plug into the wiper switch.

While your there you might like to cut some wire for the fuel computer buttons. These run from under the centre console to the passenger side. You will require the following wire colours:

Brown/Red

Brown/White

Brown/Yellow

# Instalation

Remove the steering cowling and slide out the old  wiper switch.

You will see that there are 2 black/yellow wires. Connect a DVM between one of these and ground.

Turn the front wipers on in intermittant mode and locate the black/yellow wire which is positive.

Cut this wire allowing connections to be made to both ends.

Remove the old cavalier wiper switch by sliding it out and disconnecting it.

Connect one end of the cut wire to the new wiper switch black/yellow wire.

Connect the other end of the cut wire to the new wiper switch black/white wire.

The 3 remaining brown wires are for the fuel computer buttons.

Run these 3 wires to the rear of the fuel computer and connect them up.

You will probably have the remove the dash and centre airvents to do this.

# Wiper Switch.	Fuel computer end	Function

Brown/Red 	to Brown/Red   		( Mode Switch – S )

Brown/White 	to Brown/White 		(Reset Switch  – R )

Brown  	 	to Brown/Yellow		(Common earth wire)

Lastly remove the old cavalier wiper relay from the fuse box and fit the omega relay in its place.

For the techincally minded the wiper switches varaible resistor varies between 690R & 5K

I bought my wiper switch from GM spares 01708 522510

Approx Prices

£30 for the switch

£5 for the relay

£5 + VAT postage.